|  | Introduction: The prevention of all accidents involving personal injury or property damage is essential to the efficient operation of the McSence Group. It is the policy of McSence Group that its operations are executed always in such a way as to ensure it fulfils its obligations to meet the requirements of the Health & Safety at Work (HSATW) etc. Act 1974 Company Policies: Microsoft Word - Health & Safety Policy HSE Videos: Click on this link for more info Ladders & Falls - Safety Training Video - Prevent Fatal Accidents on Ladders - Bing video |
| --- | --- |

….Virtual Toolbox Talks – Ladder & Step Ladders Safety….

Overview: Ladders and stepladders are not banned under health and safety law. The law calls for a sensible, proportionate approach to managing risk, and ladders can be a sensible and practical option for low-risk, short-duration tasks, although they should not automatically be your first choice. There are simple, sensible precautions you should take to stay safe when using portable leaning ladders and stepladders in the workplace. Make sure that you use the right type of ladder and that you know how to use it safely. Ladders are essential tools, yet more workers are injured in falls from ladders than from any other elevated surface – roofs, scaffolds, balconies, even stairs. Most falls happen because workers select the wrong type of ladder for their job, or they set up the ladder improperly and the ladder shifts or slips unexpectedly. Ladder Safety Awareness: What You Need to Know Not all ladder injuries happen at work. Many ladder injuries happen at home. Using a ladder puts you at risk for a ladder-related fall injury. What Causes Ladder Falls & Injuries? Ladder fall injuries are a persistent hazard both in the workplace and at home. There are five major causes of ladder fall incidents: 1. Incorrect extension ladder set up angle: In about 40% of cases, the cause of the ladder-related injury is a ladder sliding out at the base from an incorrect set up angle. Set up extension ladders at about a 75-degree angle. 2. Inappropriate ladder selection: Selection of a ladder with the proper size and duty-rating is important to avoid instability and structural failure. However, many ladder users lack knowledge of proper ladder selection. 3. Insufficient ladder inspection: Practice regular inspection and maintenance to reduce the likelihood of ladder structural failure. 4. Improper ladder use: Examples of improper ladder use include: - Overreaching while on the ladder - Carrying objects while going up or down the ladder - Applying excessive force while on the ladder 5. Lack of access to ladder safety tools and information: Small companies and individual ladder users do not typically receive training for safe use of portable ladders. These ladder users are difficult to reach, often do not have access to safety information, and lack the resources to develop or follow an effective ladder safety program. “Ladders First.”: Portable ladders are often the first tool we choose when working at elevation. This approach is known as “Ladders First.” Before choosing a portable ladder, consider a “Ladders Last” approach when other, often safer options, such as scaffolds, aerial lifts, or mobile ladder stands, are available. There are four main types of ladders: - A portable ladder is a ladder that can readily be moved or carried, with side rails joined at predetermined intervals by steps, rungs, cleats, or rear braces. When carefully selected, erected, and used, portable ladders temporarily provide a passageway to other surface levels; a means of support while performing elevated work; Portable ladders are typically commercially manufactured and constructed to meet the voluntary American National Standard for Ladders known as ANSI-ASC A14. - Fixed ladders are permanently attached to buildings, equipment, or structures and cannot be readily moved or carried. - Mobile ladder stands and platforms are movable, non-height adjustable, self-supporting ladders, which look like stairs mounted on wheels. - Job-made ladders are typically fabricated at construction worksites and are not commercially manufactured. 1. Choosing the Right Portable Ladder for Safety: Ladder choice is an essential you should cover in your safety talk. What you don’t want is workers climbing any old object to accomplish a task at height. Which portable ladder is the right one for you? When considering portable ladders, think of each as a candidate applying for your specific job. Some will be qualified to take you where you need to go, while others will not make the cut. One ladder may not be suitable for all your elevated work needs. Key considerations when selecting portable ladders include where it is important to understand that each of these considerations, to a large degree, is interconnected: - Supporting characteristics - Target work zone - Footprint - Setup area - Design style - Material - Working load - Size 2. Ladder Material Ladders are typically made from wood, fiberglass, or aluminium. If you’re on a construction site, you’re better off with a ladder made from aluminium or fiberglass. One of the advantages of aluminium ladders is that they’re lighter and easier to manoeuvre around. That makes a big difference when you’re walking back and forth between people around a worksite. However, if you’re working in a place with electrical sources, you’d be better off with a fiberglass ladder since they don’t conduct electricity.‌ 3. Ladder Height Next, you want to make sure that the ladder is tall enough for the task at hand. It’s better to have a ladder that’s the correct height versus trying to make do with whatever ladder you can locate. If you’re going to use an extension ladder, pick one that goes at least seven feet higher than the maximum contact point. That way, your workers can set the ladder up at the correct angle. If you’re using a stepladder, the reach height should be four feet above the height of the stepladder in use. 4. Ladder Duty Rating Make sure that a ladder can hold any necessary weight by checking its duty rating. Any ladder used on a worksite should have a sticker highlighting its duty rating attached to the side. In addition, workers should account for factors like body weight, clothing, protective equipment, and any tools they will have with them while using the ladder. ‌ 5. Setting Up a Ladder Before setting foot on the ladder, check it over for any structural defects like: - Broken or missing rungs - Cracked side rails - Corrosion of components - Other defective components‌ If you notice any issues that could impact the safety of workers, remove the ladder from the worksite and have it sent for repair or disposal. You don’t want anyone using a ladder that could lead to a serious injury or even death. Workers should set ladders up in places where there isn’t a lot of traffic. However, if it’s a busy site, they should set up barricades that prevent anyone from accidentally jostling them while using a ladder.‌ Colleagues must position ladders at an angle most conducive to providing them with stability as they finish the task at hand. The rule for extension ladders is to set the base 1 foot away from a structure for every four feet of ladder height. That way, workers can achieve a 75-degree working angle. Make sure your colleagues understand the importance of setting up ladders on a level, stable surface. If that’s hard to do, they should use a levelling device instead of stacking objects beneath the ladder legs. ‌ 6. Working on a Ladder Another point to note in a ladder safety virtual toolbox talk is how to handle working on a ladder. Colleagues should maintain three points of contact while working on top of a ladder. They can achieve using body parts like the hands, knees, and feet. In addition, crew members should face the ladder whenever they climb up and down. Workers should never stand any higher on a stepladder than two steps down from the top. For an extension ladder, they should never go higher than four rungs from the top. In addition, colleagues should avoid using ladders if the current weather conditions include rain, strong winds, or the possibility of lightning. Position step stools as close as possible to the object the worker needs to reach. A worker can tell whether a step stool is suitable for the job by how much they must strain to reach an object. If the crew member is constantly on their toes, they should switch to a ladder. Consider: Target work zone - Where is the work? When your elevation is different than where you need to work, you need to change your elevation to get to the target work zone. Think of the target work zone as your destination, and the portable ladder as your tool to get you there. Target work zones may be indoors or outdoors, and they are often large and complex, such as painting a multistory home. Large target work zones should always be further evaluated and subdivided into smaller, manageable target work zones to prevent dangerous overreaching when working from a ladder. Target work zones may be so large or complex that simply moving the ladder will not be enough; you need to change ladder styles or sizes to safely complete the elevated work. Understanding the location, dimensions, and unique complexities of your target work zone will help you select the best portable ladder for the job. Consider: Setup area - Will it be safe when erected? Before a portable ladder is erected, you must ensure a sound foundation is available to support the ladder and the ladder’s surroundings will be free from hazards. The setup area requires an evaluation of the physical environment below, around, and above. To prevent dangerous overreaching when working from a ladder, locate setup areas immediately below or slightly to the side of the target work zone. The conditions of the setup area may limit which ladder style you can select, if any at all. Self-inspection checklist Portable ladders - Are all ladders in good condition, joints between steps and side rails tight, all hardware and fittings securely attached, and moveable parts operating freely without binding or undue play? - Are there non-slip safety feet on all ladders except step ladders? - Are ladder rungs and steps free of grease and oil? - Are employees prohibited from placing a ladder in front of doors opening toward the ladder except when the door is blocked open, locked, or guarded? - Are employees prohibited from placing ladders on boxes, barrels, or other unstable bases? - Are employees instructed to face the ladder when ascending and descending? - Are employees prohibited from using ladders that are broken, missing steps, rungs, cleats, broken side rails, or other faulty parts? - Are employees instructed not to use the top step of ordinary stepladders as a step? - When portable rung ladders are used to gain access to elevated platforms, roofs, and the like, does the ladder always extend at least three feet above the elevated surface? - Do you require the users of portable rung or cleat-type ladders to place the base so that slipping will not occur or to lash or otherwise hold the ladder in place? - Do portable metal ladders have legible signs reading “CAUTION — Do Not Use Around Electrical Equipment” or equivalent wording? - Are the rungs of ladders uniformly spaced at 12 inches, center to center?

Who Does This Toolbox Talk Apply To? All Employees: This policy applies to all persons working for or on our behalf of the McSence Group of Companies which includes the subsidiary companies - McSence Communication Ltd, McSence Ltd, McSence Services Ltd & McSence Workspace Ltd in any capacity including but not limited to: - All employees at all levels, prospective employees, agency workers, seconded workers, temporary workers, contractors/sub-contractors, clients, agents, external consultants, volunteers, members of the public, group’s supply chain, third-party representatives and/or business partners who will be referred to in our Group policies as “all employees”. The Workplace: This policy applies to all persons working for or on our behalf of the McSence Group of Companies in any capacity at the workplace(s) as defined below which includes but not limited to: - McSence Premises, Offices, Units, Business Park, Client’s Premises, External Meeting Places, Customers’ Homes, Gardens, Sheltered Housing, Whilst On-Call, On-Duty, Emergency Cover, Working from Home including On-Line Meetings, Whilst Driving in Company Time, Working Public Areas (café’s, trains, coffee shops, buses etc) and will be referred to throughout this policy as “the workplace”.

HSE WEBSITE – see links below: - HSE – Asbestos Essentials - http://www.hse.gov.uk/pubns/guidance/a0.pdf - HSE – COSHH - https://www.hse.gov.uk/coshh/basics/whatdo.htm - HSE – Lifting Equipment at Work http://www.hse.gov.uk/pubns/indg290.htm - HSE – Manual Handling at Work: http://www.hse.gov.uk/pubns/indg143.htm - HSE – Needlestick - Sharps injuries - What you need to do (hse.gov.uk) - HSE – Painting & Coatings Frequently asked questions – Paint and coatings (hse.gov.uk) - HSE – Slips, Trips & Falls http://www.hse.gov.uk/toolbox/slips.htm - HSE – Working at Height/Ladders http://www.hse.gov.uk/pubns/indg455.pdf

|  | STAFF ZONE is our bespoke intranet for McSence colleagues with company information in one place where you if you click this link McSence ¦ Home ¦ McSence and enter passcode is staffzone123 it will take you to the following company information as follows: - Company Handbooks McSence Handbooks ¦ McSence - Company Policies: McSence Policies ¦ McSence - Health & Safety: McSence Health & Safety Information ¦ McSence |
| --- | --- |

__________________________

Company Intranet – Staff Zone: All the McSence Groups policies, procedures, handbooks are available on-line to all employees on the McSence Group’s Staff Zone Intranet via our website Login | McSence

Compliance: Failure to comply with the provisions of this Virtual Toolbox Talk may result in Disciplinary proceedings.

McSence Group - McSence Communication Ltd, McSence Ltd, McSence Services Ltd & McSence Workspace Ltd

T: 0131 454 1500 | E: mail@mcsence.co.uk | W: www.mcsence.co.uk | FB: www.facebook.com/McSenceGroup

Toolbox Talks: This Virtual Toolbox Talk will be reviewed annually and, if necessary, revised in the light of legislative or organisational changes Improvements will be made by learning from experience and the use of an established annual review. Should any amendments, revisions, or updates be made to this policy it is the responsibility of the Company Senior Management Team (SMT) to see that all relevant employees receive notice and training if necessary.

Caring   |   Dependable |   Respectful   |   Responsive | Sustainable