From: Services <services@mcsence.co.uk> 
Sent: 18 Feb 2026 14:05
To: All Trades <all.trades@mcsence.co.uk>; 
Subject: McSence Daily Toolbox Talk 18/02/2026 - Asbestos

INTERNAL COMMUNICATION

To: Property & Cleaning Services Colleagues & Sub-Contractors

| Hey team, today we're going to chat about something that might not come up every day on the job site, but it is super important: asbestos. We all want to keep ourselves safe at work, and understanding the risks associated with asbestos is a big part of that. What is Asbestos? Asbestos is a group of naturally occurring minerals that have been used in construction and manufacturing for decades. It was popular in building materials because it can resist heat, fire, and chemicals, as well as improve insulation. However, we now know that it can be very harmful to our health. Health Risks Breathing in asbestos fibres can lead to serious health issues. Here are some of the main concerns: - Asbestosis: A chronic lung condition caused by heavy exposure to asbestos, leading to scarring of lung tissue. - Lung Cancer: Much of the research links asbestos exposure to lung cancer, especially in smokers. - Mesothelioma: A rare and aggressive cancer primarily associated with asbestos exposure, affecting the lining of the lungs. How Can You Be Exposed? Exposures can happen in various situations, mainly related to construction and renovation work that involves: - Older Buildings: Asbestos was commonly used in insulation, flooring, and roofing materials before the late 1970s. If you’re working in older buildings, you must be aware. - Renovation Projects: If you're gutting or renovating a building, you may disturb asbestos-containing materials, releasing harmful fibres into the air. - Demolition Work: This work poses a high risk since structures may contain hidden asbestos materials. Identifying Asbestos You can't always see asbestos, but some common signs include: - Frayed Insulation: If pipe insulation is crumbling, it could contain asbestos. - Flooring Tiles: Older vinyl tiles might contain asbestos, especially if they were installed in the 1970s or earlier. - Popcorn Ceilings: Some of these textured ceilings may also contain asbestos if they were done in older homes or buildings. Precautions to Take Here’s how to protect yourself from asbestos exposure: - Training: Make sure you're trained to recognize and handle asbestos safely. - Personal Protective Equipment (PPE): Always wear the appropriate PPE, including respirators and protective clothing. - Don’t Disturb: If you suspect a material contains asbestos, do not touch or disturb it. Report it to a supervisor. - Work Area Safety: Ensure work areas are free of asbestos dust and use HEPA filters for air purification. Emergency Response If you believe you’ve been exposed to asbestos, take the following steps: - Notify a supervisor: Incidents must be documented and reported immediately. - Medical Attention: If you experience any symptoms, seek medical help right away. - Decontamination: Participate in decontamination procedures if you suspect exposure. Conclusion Being aware of the risks and knowing how to handle asbestos is vital for maintaining a safe workplace. Never hesitate to ask questions or voice concerns when dealing with older materials. Our health and safety are everyone's responsibility. Stay safe out there! |  |  |
| --- | --- | --- |
| Policies Find Out More Click… McSence Policies ¦ McSence. | Handbooks Find Out More Click… McSence Handbooks ¦ McSence. | Health & Safety Find Out More Click…. Staff Zone - Company Policies ¦ McSence. |

Have a good day…

Warmest regards,

Property & Cleaning Services | McSence Group 
McSence Business Park | 32 Sycamore Road, Mayfield, Midlothian, EH22 5TA

T: 0131 454 1520 | 1513 | 1514 | 1507

Office Hours: 7.30am to 4.00pm Monday to Friday

The information contained in this e-mail is sent in the strictest confidence for the addressee only. It is intended only for the use of the addressee and may contain legally privileged information. If you are not the intended recipient, any disclosure, copying, distribution or action taken in reliance on its contents is prohibited and may be unlawful. If you have received this e-mail in error you are requested to preserve its confidentiality and advise the sender of the transmission error.

It is the responsibility of the addressee to scan this e-mail and any attachments for computer viruses or other defects. The sender does not accept liability for any loss or damage of any nature, however caused, which may result directly, or indirectly, from this e-mail or any file attached.

Part of McSence Limited Group of Companies, Registered Office: McSence Business Park, 32 Sycamore Road, Mayfield, Dalkeith,EH22 5TA. Company Registered No. SC129794, Scottish Charity No. SC016874.

Caring   |   Dependable |   Respectful   |   Responsive | Sustainable